KONOMI STANDARD

Self-Defining Industrial Standards Compression v1.0

Build Prompt Specification

This document defines a compressed, machine-readable encoding of the major industrial automation standards (ISA-95, ISA-88, ISA-101, ISA-18.2, OPC-UA, MQTT/Sparkplug, Modbus) along with KPI definitions and crosswalk mappings. The meta-standard (Layer 0) is itself self-defining, meaning the same structures used to define the standard can describe any future standard.

LAYER 0 — META-STANDARD (How Standards Are Defined)

Layer 0 provides the grammar used to define every subsequent layer. All standards share a common envelope structure (STD) composed of User-Defined Types (UDTs), hierarchical levels, state machines, entities, relations, rules, and crosswalks.

STD — Standard Envelope

STD: {

id:         str,          # unique standard identifier, e.g. "ISA-95"

scope:      str,          # human-readable scope description

udt:        [UDT],        # list of user-defined types

hierarchy:  [LEVEL],      # ordered levels (L0..Ln)

states:     [STATE_MACHINE],

entities:   [ENTITY],

relations:  [RELATION],

rules:      [RULE],

crosswalk:  [CROSSWALK]   # mappings to/from other standards

}

UDT — User-Defined Type

UDT: {

name:        str,         # type name, PascalCase

base:        str|null,    # parent type (inheritance)

fields:      {name: type, ...},

methods:     [str],       # callable behaviours

constraints: [str]        # validation rules / invariants

}

LEVEL — Hierarchy Level

LEVEL: {

id:         str,          # e.g. "L0", "L3"

name:       str,          # human name

scope:      str,          # what this level governs

timescale:  str,          # characteristic time (ms, sec, min, hr, day, month)

systems:    [str],        # representative systems at this level

data_down:  [str],        # data flowing down from this level

data_up:    [str]         # data flowing up to this level

}

STATE_MACHINE

STATE_MACHINE: {

name:        str,

states:      [str],

initial:     str,

transitions: [{from: str, to: str, trigger: str, guard: str|null}]

}

ENTITY

ENTITY: {

name:     str,

udt:      str,            # reference to a UDT name

parent:   str|null,

children: [str],

tags:     {str: str}      # arbitrary key-value metadata

}

RELATION

RELATION: {

type:        contains | references | triggers | produces | consumes,

from:        str,         # source entity

to:          str,         # target entity

cardinality: str          # e.g. "1:N", "1:1", "N:M"

}

RULE

RULE: {

id:        str,

condition: str,           # predicate expression

action:    str,           # consequence

severity:  info | warn | error | fatal

}

CROSSWALK

CROSSWALK: {

from_std:    str,

from_entity: str,

to_std:      str,

to_entity:   str,

mapping:     str,         # semantic mapping description

transform:   str|null     # optional transformation expression

}

LAYER 1 — BASE UDTs (Primitives All Standards Use)

These primitive types are shared across every standard layer. They ensure consistent representation of identifiers, timestamps, quality codes, values, ranges, quantities, durations, and status codes.

Identifier Types

| Type | Format | Example |
| --- | --- | --- |
| UUID | 8-4-4-4-12 hex | 550e8400-e29b-41d4-a716-446655440000 |
| PATH | slash-separated segments | /Site/Area01/Line03/Tank |
| TAG | dot-separated, UPPER | AREA01.LINE03.TK100.LVL |
| URN | urn:<nid>:<nss> | urn:isa95:site:plant-west |

Timestamp Types

| Type | Description | Example |
| --- | --- | --- |
| ISO8601 | RFC 3339 UTC string | 2026-02-25T14:30:00.000Z |
| EPOCH_MS | Milliseconds since Unix epoch | 1772122200000 |
| OPC_FILETIME | 100-ns intervals since 1601-01-01 | 133845678000000000 |

Quality

Quality: GOOD:192 | BAD:0 | UNCERTAIN:64 | SUBSTITUTED:+16 | LIMITED:+4

Bit-mask composition:

GOOD             = 0xC0  (192)

BAD              = 0x00  (0)

UNCERTAIN        = 0x40  (64)

+SUBSTITUTED     = 0x10  (16)   # additive flag

+LIMITED         = 0x04  (4)    # additive flag

Example: GOOD+SUBSTITUTED = 192 + 16 = 208

Value

Value: {

v:    any,              # the payload (numeric, string, bool, array)

q:    Quality,          # quality bit-mask

t:    Timestamp,        # source timestamp

unit: str | null        # engineering unit (SI preferred)

}

Range

Range: {

lo:     num,            # lower bound

hi:     num,            # upper bound

lo_inc: bool,           # is lower bound inclusive?

hi_inc: bool,           # is upper bound inclusive?

unit:   str             # engineering unit

}

Quantity

Quantity: {

value:       num,       # measured value

unit:        str,       # engineering unit

uncertainty: num | null # +/- measurement uncertainty

}

Duration

Duration: {

value: num,

unit:  ms | s | min | hr | day | week | month | year

}

Status

Status: {

code:     int,          # numeric status code

name:     str,          # short name, e.g. "RUNNING"

desc:     str,          # human description

severity: info | warn | error | fatal

}

LAYER 2 — ISA-95 (Enterprise ↔ Control Integration)

ID: ISA-95  |  SCOPE: Enterprise to control integration

ISA-95 defines the interface between enterprise (ERP/MES) and control systems. It specifies equipment hierarchies, material models, personnel, process segments, production schedules, and performance reporting across five functional levels.

ISA95_Level Definitions

| Level | Name | Scope | Timescale | Systems |
| --- | --- | --- | --- | --- |
| L0 | Process | Physical processes, sensors, actuators | ms | Sensors, Actuators, Instruments |
| L1 | Basic Control | Direct digital control, PLC loops | ms–s | PLC, DCS I/O, Safety PLC |
| L2 | Supervisory | HMI, SCADA, batch sequencing | s–min | SCADA, HMI, Batch Engine |
| L3 | MES/MOM | Workflow, scheduling, tracking | min–hr–day | MES, LIMS, WMS, QMS |
| L4 | Enterprise | Business planning, logistics | day–month | ERP, SCM, PLM, CRM |

Equipment Hierarchy

Enterprise → Site → Area → WorkCenter → WorkUnit → Equipment

Enterprise:  { id, name, sites:[Site] }

Site:        { id, name, location, areas:[Area] }

Area:        { id, name, work_centers:[WorkCenter] }

WorkCenter:  { id, name, work_units:[WorkUnit] }

WorkUnit:    { id, name, equipment:[Equipment] }

Equipment:   { id, name, type, state:EquipmentState, mode:EquipmentMode }

PhysicalAsset & Equipment UDTs

PhysicalAsset: {

id:          UUID,

tag:         TAG,

name:        str,

type:        str,

location:    PATH,

parent:      UUID | null,

children:    [UUID],

properties:  {str: Value}

}

Equipment: {

id:          UUID,

tag:         TAG,

asset:       UUID,            # -> PhysicalAsset

state:       EquipmentState,

mode:        EquipmentMode,

capability:  [str]

}

EquipmentState: IDLE | RUNNING | PAUSED | STOPPED | FAULTED | MAINTENANCE

EquipmentMode:  PRODUCTION | MAINTENANCE | MANUAL | STANDBY | CHANGEOVER

Material & MaterialClass

MaterialClass: {

id:          UUID,

name:        str,

properties:  {str: Value},

subclasses:  [UUID]

}

Material: {

id:          UUID,

name:        str,

class:       UUID,            # -> MaterialClass

lot:         str,

sublot:      str | null,

quantity:    Quantity,

status:      str,

properties:  {str: Value}

}

Personnel

Personnel: {

id:          UUID,

name:        str,

role:        str,

qualification: [str],

availability:  str

}

ProcessSegment

ProcessSegment: {

id:           UUID,

name:         str,

description:  str,

duration:     Duration,

equipment:    [UUID],

material_in:  [{material: UUID, quantity: Quantity}],

material_out: [{material: UUID, quantity: Quantity}],

personnel:    [UUID],

parameters:   {str: Value}

}

ProductionSchedule & ProductionPerformance

ProductionSchedule: {

id:           UUID,

start:        Timestamp,

end:          Timestamp,

requests:     [ProductionRequest],

status:       DRAFT | RELEASED | ACTIVE | COMPLETED | CANCELLED

}

ProductionRequest: {

id:           UUID,

product:      str,

quantity:     Quantity,

priority:     int,

segment:      UUID,           # -> ProcessSegment

scheduled_start: Timestamp,

scheduled_end:   Timestamp

}

ProductionPerformance: {

id:           UUID,

schedule:     UUID,           # -> ProductionSchedule

responses:    [ProductionResponse],

timestamp:    Timestamp

}

ProductionResponse: {

id:           UUID,

request:      UUID,

actual_start: Timestamp,

actual_end:   Timestamp,

quantity:     Quantity,

status:       str,

kpis:         {str: Value}

}

DATA_FLOWS

L4 ↔ L3:  ProductionSchedule ↓  |  ProductionPerformance ↑

L3 ↔ L2:  WorkDirective ↓       |  BatchRecord, Events ↑

L2 ↔ L1:  Setpoints, Commands ↓ |  ProcessValues, Alarms ↑

L1 ↔ L0:  Control Signals ↓     |  Sensor Readings ↑

LAYER 3 — ISA-88 (Batch Control)

ISA-88 (S88) defines the physical model, procedural model, and recipe model for batch manufacturing. It introduces standardised state machines for phase, batch, and unit control.

S88_EquipmentLevel Hierarchy

S88_EquipmentLevel:

ProcessCell → Unit → EquipmentModule → ControlModule

ProcessCell: {

id:    UUID,

name:  str,

units: [Unit]

}

Unit: {

id:      UUID,

name:    str,

modules: [EquipmentModule],

state:   UnitState

}

EquipmentModule: {

id:       UUID,

name:     str,

controls: [ControlModule],

phases:   [Phase]

}

ControlModule: {

id:       UUID,

name:     str,

type:     VALVE | MOTOR | SENSOR | REGULATOR | DISCRETE,

state:    str,

commands: [str]

}

S88_RecipeLevel Hierarchy

S88_RecipeLevel:

GeneralRecipe → SiteRecipe → MasterRecipe → ControlRecipe

Recipe & Procedural Model

Recipe: {

id:         UUID,

name:       str,

level:      GENERAL | SITE | MASTER | CONTROL,

version:    str,

formula:    Formula,

procedure:  Procedure,

equipment:  [UUID],        # required equipment

approved:   bool,

author:     str

}

Procedure: {

id:              UUID,

name:            str,

unit_procedures: [UnitProcedure]

}

UnitProcedure: {

id:         UUID,

name:       str,

unit:       UUID,          # -> Unit

operations: [Operation]

}

Operation: {

id:     UUID,

name:   str,

phases: [Phase]

}

Phase: {

id:         UUID,

name:       str,

type:       str,

state:      PhaseState,

parameters: {str: Value},

duration:   Duration | null

}

Formula & Batch

Formula: {

inputs:     [{material: UUID, quantity: Quantity}],

outputs:    [{material: UUID, quantity: Quantity}],

parameters: {str: Value},

process:    {str: Range}     # process limits

}

Batch: {

id:          UUID,

recipe:      UUID,          # -> Recipe (ControlRecipe)

lot:         str,

state:       BatchState,

start:       Timestamp,

end:         Timestamp | null,

actual:      Formula,       # actual consumed/produced

events:      [Event],

unit:        UUID

}

PhaseState State Machine

PhaseState State Machine

========================

States: IDLE, RUNNING, COMPLETE, PAUSING, PAUSED, HOLDING, HELD,

RESTARTING, STOPPING, STOPPED, ABORTING, ABORTED

Initial: IDLE

Transitions:

IDLE       --(Start)-->      RUNNING

RUNNING    --(complete)-->   COMPLETE

RUNNING    --(Hold)-->       HOLDING

HOLDING    --(held)-->       HELD

HELD       --(Restart)-->    RESTARTING

RESTARTING --(running)-->    RUNNING

RUNNING    --(Pause)-->      PAUSING

PAUSING    --(paused)-->     PAUSED

PAUSED     --(Resume)-->     RUNNING

RUNNING    --(Stop)-->       STOPPING

HOLDING    --(Stop)-->       STOPPING

HELD       --(Stop)-->       STOPPING

PAUSED     --(Stop)-->       STOPPING

STOPPING   --(stopped)-->    STOPPED

STOPPED    --(Reset)-->      IDLE

COMPLETE   --(Reset)-->      IDLE

*any*      --(Abort)-->      ABORTING

ABORTING   --(aborted)-->    ABORTED

ABORTED    --(Reset)-->      IDLE

BatchState State Machine

BatchState State Machine

========================

States: NOT_STARTED, IN_PROGRESS, COMPLETE, SUSPENDED, CANCELLED, ABORTED

Initial: NOT_STARTED

Transitions:

NOT_STARTED  --(Start)-->     IN_PROGRESS

IN_PROGRESS  --(complete)-->  COMPLETE

IN_PROGRESS  --(Suspend)-->   SUSPENDED

SUSPENDED    --(Resume)-->    IN_PROGRESS

IN_PROGRESS  --(Cancel)-->    CANCELLED

SUSPENDED    --(Cancel)-->    CANCELLED

IN_PROGRESS  --(Abort)-->     ABORTED

SUSPENDED    --(Abort)-->     ABORTED

UnitState (PackML) State Machine

UnitState (PackML) State Machine

================================

States: IDLE, STARTING, EXECUTE, COMPLETING, COMPLETE,

HOLDING, HELD, UNHOLDING,

SUSPENDING, SUSPENDED, UNSUSPENDING,

STOPPING, STOPPED, ABORTING, ABORTED, CLEARING, RESETTING

Initial: STOPPED

Transitions:

STOPPED      --(Reset)-->       RESETTING

RESETTING    --(done)-->        IDLE

IDLE         --(Start)-->       STARTING

STARTING     --(done)-->        EXECUTE

EXECUTE      --(complete)-->    COMPLETING

COMPLETING   --(done)-->        COMPLETE

COMPLETE     --(Reset)-->       RESETTING

EXECUTE      --(Hold)-->        HOLDING

HOLDING      --(done)-->        HELD

HELD         --(Unhold)-->      UNHOLDING

UNHOLDING    --(done)-->        EXECUTE

EXECUTE      --(Suspend)-->     SUSPENDING

SUSPENDING   --(done)-->        SUSPENDED

SUSPENDED    --(Unsuspend)-->   UNSUSPENDING

UNSUSPENDING --(done)-->        EXECUTE

*any*        --(Stop)-->        STOPPING

STOPPING     --(done)-->        STOPPED

*any*        --(Abort)-->       ABORTING

ABORTING     --(done)-->        ABORTED

ABORTED      --(Clear)-->      CLEARING

CLEARING     --(done)-->        STOPPED

LAYER 4 — ISA-101 (HMI Design)

ISA-101 defines principles and practices for effective Human-Machine Interface design in industrial environments, focusing on situation awareness, consistency, and safety.

HMI_Principles

HMI_Principles:

1. Situation Awareness   – Operator must understand current state at a glance

2. Consistency           – Uniform colour, layout, and interaction patterns

3. Simplicity            – Show only what is needed for the task

4. Effective Alarming    – Integrate with ISA-18.2; no orphan alarms on HMI

5. User-Centered Design  – Design for the operator, not the engineer

6. Performance           – Response time < 1s for navigation, < 250ms for value update

7. Accessibility         – Readable under all lighting; colour-blind safe palette

8. Security              – Role-based access; audit trail for commands

HMI_Layer (Navigation Levels)

| Layer | Name | Scope | Content |
| --- | --- | --- | --- |
| L1 | Overview | Entire plant / enterprise | KPI tiles, area status, major alarms |
| L2 | Area | Production area / zone | Equipment status, throughput, area alarms |
| L3 | Unit | Single unit / machine | P&ID, operating parameters, controls |
| L4 | Detail | Loop / device detail | Faceplate, trend, tuning, diagnostics |
| L5 | Support | Diagnostics & maintenance | Calibration, network status, logs |

ColorMeaning

| State | Hex Code | Color | Usage |
| --- | --- | --- | --- |
| Normal | #808080 | Gray | Equipment in normal/idle state |
| Running | #00B050 | Green | Equipment actively running |
| Stopped | #FFFFFF | White | Equipment intentionally stopped |
| Warning | #FFC000 | Amber/Yellow | Approaching limit or advisory |
| Alarm | #FF0000 | Red | Active alarm requiring attention |
| Fault | #FF0000 | Red (flashing) | Equipment fault or failure |
| Maintenance | #0070C0 | Blue | Equipment in maintenance mode |
| Disabled | #A6A6A6 | Light Gray | Equipment disabled / out of service |
| Manual | #FFC000 | Amber | Equipment in manual override |
| Transition | #FFFF00 | Yellow | Equipment transitioning between states |

GraphicElement, Faceplate, Trend

GraphicElement: {

id:       UUID,

type:     VESSEL | VALVE | PUMP | MOTOR | CONVEYOR | PIPE | INDICATOR | TEXT | SHAPE,

tag:      TAG,

state:    str,

color:    ColorMeaning,

position: {x: num, y: num},

size:     {w: num, h: num},

layer:    int,            # z-order

tooltip:  str

}

Faceplate: {

tag:        TAG,

title:      str,

pv:         Value,        # process value

sp:         Value,        # setpoint

out:        Value,        # output

mode:       AUTO | MANUAL | CASCADE | RATIO,

alarms:     [Alarm],

trend_link: UUID

}

Trend: {

id:       UUID,

title:    str,

pens:     [{tag: TAG, color: str, scale: Range, axis: LEFT|RIGHT}],

timespan: Duration,

refresh:  Duration

}

Rules R1–R8

R1: Every process value must show engineering units

R2: Color must not be the sole indicator of state (use shape/text also)

R3: Navigation depth ≤ 3 clicks from Overview to Detail

R4: Alarm banners visible on every screen

R5: Faceplate must show PV, SP, OUT, MODE, and active alarms

R6: Trend pens limited to 6 per chart for readability

R7: Animation rate ≤ 1 Hz to avoid distraction

R8: Critical controls require confirmation dialog

LAYER 5 — ISA-18.2 (Alarm Management)

ISA-18.2 provides a lifecycle model for alarm management, covering identification, rationalization, design, implementation, operation, maintenance, monitoring, and change management.

AlarmPriority

| Priority | Name | Response Time | Color | Description |
| --- | --- | --- | --- | --- |
| P1 | Critical | < 5 min | #FF0000 (Red) | Immediate safety/environmental risk |
| P2 | High | < 10 min | #FF8000 (Orange) | Significant operational impact |
| P3 | Medium | < 30 min | #FFC000 (Amber) | Moderate operational impact |
| P4 | Low | < 60 min | #FFFF00 (Yellow) | Advisory; awareness only |

AlarmState

AlarmState:

NORM          – Normal (no alarm condition, acknowledged)

UNACK         – Active and unacknowledged

ACKED         – Active and acknowledged

RTN_UNACK     – Returned to normal but not yet acknowledged

SHELVED       – Temporarily suppressed (time-limited)

OUT_OF_SERVICE – Disabled by authorized personnel

AlarmLifecycle State Machine

AlarmLifecycle State Machine

============================

States: NORM, UNACK, ACKED, RTN_UNACK, SHELVED, OUT_OF_SERVICE

Initial: NORM

Transitions:

NORM       --(alarm condition)-->       UNACK

UNACK      --(Acknowledge)-->           ACKED

UNACK      --(condition clears)-->      RTN_UNACK

ACKED      --(condition clears)-->      NORM

RTN_UNACK  --(Acknowledge)-->           NORM

NORM       --(Shelve)-->                SHELVED

UNACK      --(Shelve)-->                SHELVED

ACKED      --(Shelve)-->                SHELVED

SHELVED    --(Unshelve / timer)-->      NORM

*any*      --(Disable)-->               OUT_OF_SERVICE

OUT_OF_SERVICE --(Enable)-->            NORM

Guards:

Shelve requires: reason, duration (max 24h default), authorization

OUT_OF_SERVICE requires: MOC approval, documented justification

Alarm UDT

Alarm: {

id:            UUID,

tag:           TAG,

name:          str,

description:   str,

priority:      P1 | P2 | P3 | P4,

class:         str,             # -> AlarmClass

type:          HI | HIHI | LO | LOLO | DEV | RATE | DISC | DIAG,

setpoint:      Value,           # alarm trip point

deadband:      num,             # hysteresis

delay:         Duration,        # on-delay before annunciation

state:         AlarmState,

timestamp:     Timestamp,       # time of last state change

ack_required:  bool,

shelve_max:    Duration,

consequence:   str,             # what happens if not responded to

response:      str,             # recommended operator action

area:          PATH,

equipment:     UUID

}

AlarmClass

AlarmClass: {

name:          str,             # e.g. "PROCESS", "EQUIPMENT", "SAFETY"

description:   str,

priorities:    [P1|P2|P3|P4],   # allowed priorities

ack_required:  bool,

logging:       bool,

notification:  [str]            # channels: HMI, EMAIL, SMS, HORN, BEACON

}

Rationalization Rules R1–R7

R1: Every alarm must have a documented consequence and response

R2: Every alarm must have a unique, meaningful tag and description

R3: No duplicate alarms (same condition on same measurement)

R4: Priority distribution target: P1 < 5%, P2 < 15%, P3 < 30%, P4 remainder

R5: Standing alarms (active > 24h) must be reviewed and resolved

R6: Alarm rate target: ≤ 6 alarms per operator per hour (avg over 30 days)

R7: Chattering alarms (> 5 activations in 1 min) must be re-engineered

Metrics

AlarmRate:    { value: num, unit: "alarms/operator/hour", target: "≤ 6" }

FloodRate:    { value: num, unit: "alarms/10min", threshold: "> 10 = flood" }

StaleAlarms:  { count: int, definition: "active > 24 hours without change" }

Chattering:   { count: int, definition: "> 5 state changes in 1 minute" }

BadActors:    { top_n: int, definition: "most frequent alarms in period" }

SuppressionPct: { value: num, unit: "%", definition: "shelved + OOS / total" }

ResponseTime:   { p50: Duration, p95: Duration, by_priority: {P1:Duration, ...} }

LAYER 6 — OPC-UA (Communication)

OPC Unified Architecture is a platform-independent, service-oriented architecture for secure and reliable data exchange in industrial automation.

OPC_NodeClass & OPC_Node

OPC_NodeClass:

OBJECT | VARIABLE | METHOD | OBJECT_TYPE | VARIABLE_TYPE |

REFERENCE_TYPE | DATA_TYPE | VIEW

OPC_Node: {

node_id:      str,            # e.g. "ns=2;s=Tank1.Level"

browse_name:  str,

display_name: str,

node_class:   OPC_NodeClass,

description:  str,

references:   [{ref_type: str, target: str, forward: bool}]

}

OPC_Variable

OPC_Variable: {

node_id:       str,

data_type:     str,           # OPC built-in type

value:         Value,

access_level:  READ | WRITE | READ_WRITE | HISTORY_READ,

historizing:   bool,

eu_range:      Range | null,

engineering_units: str | null,

min_sampling_interval: Duration

}

OPC_Method

OPC_Method: {

node_id:        str,

input_args:     [{name: str, data_type: str, description: str}],

output_args:    [{name: str, data_type: str, description: str}],

executable:     bool,

user_executable: bool

}

OPC_Subscription & OPC_MonitoredItem

OPC_Subscription: {

id:                    int,

publishing_interval:   Duration,

lifetime_count:        int,

max_keepalive_count:   int,

max_notifications:     int,

priority:              int,

monitored_items:       [OPC_MonitoredItem]

}

OPC_MonitoredItem: {

item_id:             int,

node_id:             str,

sampling_interval:   Duration,

queue_size:          int,

discard_oldest:      bool,

filter:              str | null,

deadband_type:       NONE | ABSOLUTE | PERCENT,

deadband_value:      num

}

ADDRESS_SPACE Hierarchy

ADDRESS_SPACE:

Root

├── Objects

│   ├── Server          (ServerStatus, Diagnostics, Capabilities)

│   ├── DeviceSet       (companion-spec managed)

│   └── <Custom>        (application objects)

├── Types

│   ├── ObjectTypes

│   ├── VariableTypes

│   ├── DataTypes

│   └── ReferenceTypes

└── Views               (filtered perspectives)

COMPANION_SPECS

COMPANION_SPECS:

OPC-UA for ISA-95     – Equipment, Material, Production models

OPC-UA for PackML     – State machine, admin modes

OPC-UA for Robotics   – Robot motion, joints, DI

OPC-UA for CNC        – Machine tools, programs, channels

OPC-UA for Weighing   – Scale data model

OPC-UA for Vision     – Camera, recipe, result

OPC-UA for MDIS       – Subsea/offshore devices

OPC-UA for PLCopen    – Motion function blocks

LAYER 7 — MQTT / Sparkplug (Messaging)

MQTT is a lightweight publish/subscribe messaging protocol. Sparkplug B defines a topic namespace and payload encoding for SCADA/IIoT use on top of MQTT, adding birth/death semantics and rich data typing.

MQTT_QoS

| Level | Name | Guarantee | Use Case |
| --- | --- | --- | --- |
| QoS 0 | At most once | Fire-and-forget; no ACK | Telemetry, high-frequency sensors |
| QoS 1 | At least once | ACK'd; possible duplicates | Alarms, events, commands |
| QoS 2 | Exactly once | 4-step handshake; no duplicates | Financial, safety-critical |

MQTT_Topic Format

MQTT_Topic:

Format:  {namespace}/{group_id}/{message_type}/{edge_node_id}[/{device_id}]

Example: spBv1.0/PlantWest/DDATA/EdgeNode01/PLC_Line3

Wildcards:

+ = single level   e.g. spBv1.0/+/DDATA/#

# = multi level    e.g. spBv1.0/PlantWest/#

Sparkplug_Topic Message Types

| Message Type | Direction | Purpose |
| --- | --- | --- |
| NBIRTH | Edge → Broker | Node birth certificate; full metric list with metadata |
| NDEATH | Broker (LWT) | Node death; last-will-and-testament on disconnect |
| DBIRTH | Edge → Broker | Device birth certificate; full metric list for device |
| DDEATH | Edge → Broker | Device death notification |
| NDATA | Edge → Broker | Node data; changed metrics only (report by exception) |
| DDATA | Edge → Broker | Device data; changed metrics only |
| NCMD | App → Edge | Node command; write values to edge node |
| DCMD | App → Edge | Device command; write values to specific device |

Sparkplug_Payload & Sparkplug_DataType

Sparkplug_Payload: {

timestamp:  EPOCH_MS,

seq:        int (0-255),

metrics:    [{

name:       str,           # dot-path, e.g. "Motors/Pump1/Speed"

alias:      int | null,    # numeric alias (after BIRTH)

timestamp:  EPOCH_MS,

datatype:   Sparkplug_DataType,

value:      any,

is_null:    bool,

metadata:   {description: str, unit: str, ...} | null,

properties: {str: any} | null

}]

}

Sparkplug_DataType:

Int8, Int16, Int32, Int64,

UInt8, UInt16, UInt32, UInt64,

Float, Double,

Boolean, String,

DateTime (EPOCH_MS),

Text (UTF-8),

UUID,

Bytes,

DataSet,

Template

Rules R1–R5

R1: Every Edge Node must publish NBIRTH on connect with full metric schema

R2: MQTT broker must be configured with NDEATH as LWT for every edge node

R3: After BIRTH, NDATA/DDATA should use aliases (not names) for efficiency

R4: Sequence numbers must increment 0-255 and roll over; gap = data loss

R5: SCADA/application must subscribe to STATE topic to detect broker status

LAYER 8 — Modbus (Field Protocol)

Modbus is a serial/TCP master-slave protocol for communicating with PLCs and field instruments. It defines four register types and a set of function codes for reading/writing data.

Modbus_Register

| Type | Access | Address Range | Size | Function Codes (Read/Write) |
| --- | --- | --- | --- | --- |
| Coil | R/W | 00001–09999 | 1 bit | FC01 / FC05, FC15 |
| Discrete Input | R | 10001–19999 | 1 bit | FC02 / — |
| Holding Register | R/W | 40001–49999 | 16 bit | FC03 / FC06, FC16 |
| Input Register | R | 30001–39999 | 16 bit | FC04 / — |

Modbus_DataType

| Type | Size (registers) | Byte Order | Description |
| --- | --- | --- | --- |
| BOOL | 1 bit / coil | — | Single bit (coil or packed) |
| INT16 | 1 | Big-endian | Signed 16-bit integer |
| UINT16 | 1 | Big-endian | Unsigned 16-bit integer |
| INT32 | 2 | Big-endian / ABCD | Signed 32-bit integer |
| UINT32 | 2 | Big-endian / ABCD | Unsigned 32-bit integer |
| FLOAT32 | 2 | Big-endian / ABCD | IEEE 754 float |
| FLOAT64 | 4 | Big-endian / ABCD | IEEE 754 double |
| STRING | N/2 | 2 chars/register | ASCII string, null-padded |

Modbus_Map

Modbus_Map: {

device:     str,              # device name or model

slave_id:   int (1-247),

registers:  [{

name:       str,

address:    int,          # 0-based register address

type:       Modbus_DataType,

reg_type:   COIL | DISCRETE | HOLDING | INPUT,

scale:      num,          # raw * scale = engineering value

offset:     num,          # + offset after scaling

unit:       str,

access:     R | RW,

description: str

}]

}

FUNCTION_CODES

| Code | Name | Description |
| --- | --- | --- |
| FC01 | Read Coils | Read 1-2000 coil statuses |
| FC02 | Read Discrete Inputs | Read 1-2000 discrete input statuses |
| FC03 | Read Holding Registers | Read 1-125 holding registers |
| FC04 | Read Input Registers | Read 1-125 input registers |
| FC05 | Write Single Coil | Write one coil (ON=0xFF00, OFF=0x0000) |
| FC06 | Write Single Register | Write one holding register |
| FC15 | Write Multiple Coils | Write 1-1968 coils |
| FC16 | Write Multiple Registers | Write 1-123 holding registers |
| FC23 | Read/Write Multiple Registers | Atomic read+write in one transaction |

EXCEPTION_CODES

| Code | Name | Meaning |
| --- | --- | --- |
| 01 | ILLEGAL_FUNCTION | Function code not supported by device |
| 02 | ILLEGAL_DATA_ADDRESS | Register address out of valid range |
| 03 | ILLEGAL_DATA_VALUE | Value in request data field is invalid |
| 04 | SLAVE_DEVICE_FAILURE | Unrecoverable error on slave device |
| 05 | ACKNOWLEDGE | Request accepted; processing in progress (long op) |
| 06 | SLAVE_DEVICE_BUSY | Slave busy; retry later |
| 08 | MEMORY_PARITY_ERROR | Parity error detected in extended memory |
| 0A | GATEWAY_PATH_UNAVAIL | Gateway misconfigured or target unreachable |
| 0B | GATEWAY_TARGET_FAILED | Target device failed to respond via gateway |

LAYER 9 — KPIs (Performance Metrics)

Standard key performance indicators for manufacturing operations, rooted in the OEE (Overall Equipment Effectiveness) framework.

OEE — Overall Equipment Effectiveness

OEE: {

formula:      "Availability * Performance * Quality",

target:       ">= 0.85 (world-class)",

availability: { formula: "RunTime / PlannedProductionTime",   target: ">= 0.90" },

performance:  { formula: "IdealCycleTime * TotalPieces / RunTime", target: ">= 0.95" },

quality:      { formula: "GoodPieces / TotalPieces",          target: ">= 0.99" }

}

Supporting KPIs

| KPI | Formula / Definition | Unit | Target |
| --- | --- | --- | --- |
| MTBF | TotalUpTime / NumberOfBreakdowns | hours | > category baseline |
| MTTR | TotalRepairTime / NumberOfRepairs | hours | < category baseline |
| Downtime | PlannedProductionTime - RunTime | hours | Minimize |
| FirstPassYield | GoodFirstTime / TotalProduced | % | >= 95% |
| CycleTime | EndTime - StartTime per unit | seconds | <= Ideal |
| Throughput | Units produced per time period | units/hr | >= target |
| EnergyKPI | Energy consumed per unit produced | kWh/unit | Minimize |

KPI Tree

KPI Tree:

OEE

├── Availability

│   ├── PlannedDowntime (changeover, maintenance)

│   ├── UnplannedDowntime (breakdowns, faults)

│   ├── MTBF

│   └── MTTR

├── Performance

│   ├── IdealCycleTime

│   ├── ActualCycleTime

│   ├── SpeedLoss

│   ├── SmallStops (< 5 min)

│   └── Throughput

└── Quality

├── FirstPassYield

├── Scrap / Rework

├── Defect rate

└── GoodPieces / TotalPieces

CROSSWALKS (δ maps)

Crosswalks define semantic mappings between entities in different standards, enabling interoperability and data translation across systems.

ISA-95 ↔ ISA-88

| ISA-95 Entity | ISA-88 Entity | Mapping | Transform |
| --- | --- | --- | --- |
| Area | ProcessCell | 1:1 containment | — |
| WorkCenter | Unit | 1:1 containment | — |
| WorkUnit | EquipmentModule | 1:N containment | — |
| Equipment | ControlModule | 1:1 identity | — |
| ProcessSegment | Procedure | Semantic equivalence | Map parameters↔formula |
| ProductionRequest | Batch | 1:1 triggering | Schedule→Recipe binding |
| ProductionResponse | BatchRecord | 1:1 produces | Actuals→KPIs |
| EquipmentState | UnitState(PackML) | State mapping | See PackML crosswalk |

ISA-95 ↔ OPC-UA

| ISA-95 Entity | OPC-UA Entity | Mapping | Transform |
| --- | --- | --- | --- |
| Enterprise | Root/Objects | Container | Namespace 0 |
| Site | FolderType node | Container | Namespace per site |
| Area | FolderType node | Container | — |
| Equipment | BaseObjectType | 1:1 type instance | Custom ObjectType |
| EquipmentState | StateVariableType | State mapping | Enum→UA State |
| Material | BaseObjectType | 1:1 instance | MaterialType ObjectType |
| ProcessValue | OPC_Variable | 1:1 mapping | Value→UA DataValue |

ISA-88 ↔ PackML

| ISA-88 State | PackML State | Notes |
| --- | --- | --- |
| IDLE | IDLE | Direct mapping |
| RUNNING | EXECUTE | Rename; semantically equivalent |
| PAUSING | SUSPENDING | S88 PAUSE ≈ PackML SUSPEND |
| PAUSED | SUSPENDED | Held vs Suspended distinction |
| HOLDING | HOLDING | Direct mapping |
| HELD | HELD | Direct mapping |
| COMPLETE | COMPLETE | Direct mapping |
| STOPPING | STOPPING | Direct mapping |
| STOPPED | STOPPED | Direct mapping |
| ABORTING | ABORTING | Direct mapping |
| ABORTED | ABORTED | Direct mapping |

ISA-101 ↔ ISA-18.2

| ISA-101 Element | ISA-18.2 Element | Mapping | Transform |
| --- | --- | --- | --- |
| AlarmBanner | Alarm (UNACK) | Display filter | Priority→color, sort by time |
| ColorMeaning:Alarm | AlarmPriority | Color mapping | P1→Red, P2→Orange, P3→Amber, P4→Yellow |
| Faceplate.alarms | Alarm[] | Tag filter | Filter alarms by faceplate tag |
| HMI_Layer:L1 | AlarmSummary | Aggregation | Count by priority and area |
| HMI_Layer:L4 | AlarmDetail | Detail view | Full alarm properties and history |
| GraphicElement | AlarmState | Visual encoding | State→animation (flash/steady) |

OPC-UA ↔ Sparkplug

| OPC-UA Entity | Sparkplug Entity | Mapping | Transform |
| --- | --- | --- | --- |
| OPC_Node | Metric | 1:1 mapping | NodeId→metric name path |
| OPC_Variable.value | Metric.value | Value mapping | UA DataValue→SP value+quality |
| OPC_Variable.type | Sparkplug_DataType | Type mapping | UA built-in type→SP DataType enum |
| OPC_Subscription | MQTT_Subscription | Pub/sub mapping | MonitoredItem→Topic+QoS |
| OPC_Method | NCMD/DCMD | Command mapping | Method call→CMD message |
| AddressSpace tree | Metric name hierarchy | Namespace mapping | Browse path→dot-path name |

USAGE

The KONOMI Standard is designed for programmatic consumption. Below are Python API examples demonstrating core operations.

Parse — Load a compressed standard definition

from konomi import parse

std = parse("ISA-95")

print(std.id)           # "ISA-95"

print(std.scope)        # "enterprise to control integration"

print(std.hierarchy)    # [L0, L1, L2, L3, L4]

print(std.entities)     # [Enterprise, Site, Area, ...]

Expand — Inflate compressed representation to full objects

from konomi import expand

full = expand(std)

for entity in full.entities:

print(f"{entity.name}: {entity.udt.fields}")

# Enterprise: {id: UUID, name: str, sites: [Site]}

# Site: {id: UUID, name: str, location: str, areas: [Area]}

# ...

Validate — Check a data instance against a standard

from konomi import validate

equipment = {

"id": "550e8400-...",

"tag": "AREA01.LINE03.PUMP01",

"state": "RUNNING",

"mode": "PRODUCTION"

}

result = validate(equipment, std, entity="Equipment")

print(result.valid)     # True

print(result.errors)    # []

print(result.warnings)  # ["Consider adding 'capability' field"]

Crosswalk — Map entities between standards

from konomi import crosswalk

# Map an ISA-95 Area to its ISA-88 equivalent

mapped = crosswalk(

entity=my_area,

from_std="ISA-95",

to_std="ISA-88"

)

print(mapped.entity)    # "ProcessCell"

print(mapped.transform) # "1:1 containment"

# Translate OPC-UA variable to Sparkplug metric

metric = crosswalk(

entity=opc_variable,

from_std="OPC-UA",

to_std="Sparkplug"

)

print(metric.name)      # "Motors.Pump1.Speed"

print(metric.datatype)  # "Double"

Generate — Create code/config from standard definitions

from konomi import generate

# Generate OPC-UA NodeSet XML from equipment hierarchy

xml = generate(std, format="opcua-nodeset", scope="Equipment")

with open("nodeset.xml", "w") as f:

f.write(xml)

# Generate Sparkplug-compatible tag list

tags = generate(std, format="sparkplug-tags", scope="Area01")

# [{"name": "Area01.Line03.Tank.Level", "datatype": "Double", ...}, ...]

# Generate Modbus map from standard tag definitions

modbus = generate(std, format="modbus-map", device="PLC_Line3")

# {"slave_id": 1, "registers": [...]}

FINAL GOAL

The KONOMI Standard compresses the essential structure of major industrial automation standards into a single, self-defining, machine-readable format. The layered architecture ensures each concern is addressed at the appropriate level of abstraction:

LAYER 0  META-STANDARD     Grammar for defining any standard (STD, UDT, LEVEL,

STATE_MACHINE, ENTITY, RELATION, RULE, CROSSWALK)

LAYER 1  BASE UDTs          Shared primitives (Identifier, Timestamp, Quality,

Value, Range, Quantity, Duration, Status)

LAYER 2  ISA-95             Enterprise ↔ Control integration; equipment hierarchy,

material, personnel, scheduling, performance

LAYER 3  ISA-88             Batch control; recipe model, procedural model,

state machines (Phase, Batch, PackML Unit)

LAYER 4  ISA-101            HMI design; navigation layers, colour standards,

graphic elements, faceplates, trends

LAYER 5  ISA-18.2           Alarm management; priority, lifecycle, rationalization,

metrics, alarm UDT

LAYER 6  OPC-UA             Unified communication; address space, nodes, variables,

methods, subscriptions, companion specs

LAYER 7  MQTT/Sparkplug     Lightweight messaging; QoS, topic namespace, birth/death

certificates, payload encoding, data types

LAYER 8  Modbus             Field protocol; register types, data types, function

codes, exception codes, device mapping

LAYER 9  KPIs               Performance metrics; OEE tree, MTBF, MTTR, cycle time,

throughput, energy, first-pass yield

CROSSWALKS                  Bidirectional mappings: ISA-95↔ISA-88, ISA-95↔OPC-UA,

ISA-88↔PackML, ISA-101↔ISA-18.2, OPC-UA↔Sparkplug

AGENTS

When consumed by an AI agent or code-generation system, the KONOMI Standard enables:

AGENTS:

1. PARSING      – Ingest compressed definitions and reconstruct full object models

2. VALIDATION   – Check any data instance against the relevant standard layer

3. CROSSWALKING – Translate entities and values between any two mapped standards

4. GENERATION   – Produce OPC-UA NodeSets, Sparkplug tag lists, Modbus maps,

HMI screen specs, alarm databases, and KPI dashboards

5. REASONING    – Answer questions about standard compliance, state transitions,

data flows, and interoperability

6. DESIGN       – Propose compliant system architectures using the standard

definitions as constraints

The self-defining nature of Layer 0 means new standards (e.g., ISA-62443 for cybersecurity, IEC 61850 for power systems) can be added using the same meta-grammar without modifying the framework itself.